PC6B Power Combo Instructions

1. Introduction:

The PC6B is a power combo unit, which is designed for

DT 2 wire system to supply power for outdoor station,

indoor monitor and other accessories. It can work both

as a normal power combo and auxiliary power combo. The

features are as follows:

· Universal AC input/full range

· Multi protection: short circuit, overload, over voltage

· Integrated with DT-DPS(power separator), Additional

DT-DPS is not needed.

· Support up to 4~8 monitors(depend on models)

4. Wiring:

Open the AC cover:

1. push the cover bottom

towards the center with

your hand;

2. turn over and pull it out.

After connect the AC cable,

it is suggested to close the cover.

AC Input

· DIN rail mounting

PG    N

L

L

CN

1 2 3

2. Terminal Description:

PG N

L

L

CN

BUS(IM) BUS(DS)

1 2 3

LED

Connect to monitor

Connect to door staiton

PG

N

L

100~240 Vac

L

Note:

1. the AC cable is not included in the package, it must

2. pin1&2 should be short-circuited with a C3-3P

line(included in the package) if the unit work as a nor-

mal power combo. But if it work as a auxiliary power

BUS(IM) BUS(DS)

PG: Earth ground port.

N: AC input.

L: AC input.

LED: power indicator,always on when plug in power.

CN: bus control terminal.

BUS(IM): indoor monitor connection terminal.

BUS(DS): door station connection terminal.

3. Mounting:

Step1: Mount the din rail to the wall with screws ;

Step2: Pull down the mounting buckle,then hang the

unit on din rail.

Input Voltage:

Input Frequency:

Rated Output Voltage:

Rated Output Current:

Working Temperature:

SW Port Input Voltage:

Dimension:

6. Warning:

100~240Vac

50~60Hz

DC 26V+2V

1.5A

-200C~550C

Max.230Vac, 2A

140*90*60mm

Mounting Buckle

Din rail

Din rail

- The unit can only be used in DT 2 wire products sup-

plied by our company.

- Don’t connect the unit with any non-speci ed power

source. Fire or electric shock could result.

- Don’t dismantle or alter the unit. Fire or electric shock

could result.

- The unit must be installed and wired by a quali ed

technician.

- Keep the unit away from water or any other liquid. Fire

or electric shock could result.

*100~240Vac